1) Time

Digital Design and Computer Architecture

Harris and Harris, © Elsevier, 2012

Lab 1 Solutions

2) Truth Table

| Inputs |  |  | Outputs |  |
| --- | --- | --- | --- | --- |
| Cin | B | A | Cout | S |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 1 |
| 0 | 1 | 0 | 0 | 1 |
| 0 | 1 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 |
| 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 1 | 1 |

Completed Truth Table for Full Adder

3) Alatera Schematic

Note: the minimal number of gates reuses one of the XOR gates in the SUM logic: SUM = A XOR B XOR C, COUT = (A XOR B)CIN OR (AB).  Full credit will be given for either solution.

4) Simulation Waveforms

5) DE2 board should pass all eight tests

6) Breadboard should pass all eight tests