Lab 2-	Unshielded Twisted Pair (UTP) Lab		Name:

Objective:  	The objective of this lab is for the student to gain an understanding of the

following:

- Learn how to make RJ-45 patch cables

- Explore how to test the patch cables

- Learn how to make RJ-45 crossover cables

- Explore how to test the crossover cables

- Explore how to test the LANs horizontal cabling

Review the steps for terminating twisted pair cable presented in Chapter 2 prior to starting the lab.  Also make sure you know what tools and test equipment you have available in the lab.

Test Equipment Required

- UTP Cable termination kit

- Fluke MicroScanner² Cable Verifier

- Twisted Pair Cable

- RJ-45 Pugs

Required:	Each student is required to complete the following tasks:

1.  make one patch cable ~ 3ft. (terminate both ends)		_____

Your cable must be wire for T568A.

Refer to section 2-4 in the text for the steps for terminating the cable.

Lab 2-	Unshielded Twisted Pair (UTP) Lab						page 2/2

2.  make one crossover cable	~3ft.  (terminate both ends)		_____

Refer to section 2-4 in the text for the steps for terminating the cable.

3   Test the patch cable and the crossover cables to verify they have been 	  	     properly assembled.						 _____

Note: This step requires the use of a cable tester.

Note:  Have the instructor initial that you have completed the task!

Assignment

- Your boss wants to know if a UTP cable that is wired as follows will work.

1-------3

2-------6

3-------1

4-------7

5-------8

6-------2

7-------4

8-------5

Prepare a memo to the boss commenting on his question.  Justify your answer.  	     This is not a yes/no question. Explain your answer in detail.  You will submit 		     your memo to your instructor.  Remember, this treat this as a memo to your 		     boss.

This memo should be formally prepared as a “memo to the boss.”  The student should point out that this cable is not properly wired although it will work for 10/100 Mbps but will not work for 1/10G.  It also might have problems with Power over Ethernet but the student might not be aware of this technology yet.