# Chapter 2 Circuit Elements

## Exercises

Exercise 2.4-1 Find the power absorbed by a 100-ohm resistor when it is connected directly across a constant 10-V source.

Answer: 1-W

### Solution:

Exercise 2.4-2 A voltage source v = 10 cos t V is connected across a resistor of 10 ohms. Find the power delivered to the resistor.

Answer: 10 cos2 t W

### Solution:

Exercise 2.7-1 Find the power absorbed by the CCCS in Figure E 2.7-1.

Hint: The controlling element of this dependent source is a short circuit. The voltage across a short circuit is zero. Hence, the power absorbed by the controlling element is zero. How much power is absorbed by the controlled element?

Answer:–115.2 watts are absorbed by the CCCS. (The CCCS delivers +115.2 watts to the rest of the circuit.)

Figure E 2.7-1

### Solution:

|  | ##### id and vd adhere to the passive convention so ##### is the power received by the dependent source |
| --- | --- |

Exercise 2.8-1 For the potentiometer circuit of Figure 2.8-2, calculate the meter voltage, vm, when θ = 45°, Rp = 20 kΩ, and I = 2 mA.

Answer: vm = 5 V

Figure 2.8-2

### Solution:

| Exercise 2.8-2 The voltage and current of an AD590 temperature sensor of Figure 2.8-3 are 10 V and 280 μA, respectively. Determine the measured temperature. Answer: T = 280°K, or approximately 6.85°C | Figure 2.8-3 |
| --- | --- |

### Solution:

| Exercise 2.9-1 What is the value of the current i in Figure E 2.9-1 at time t = 4 s? Answer: i = 0 amperes at t = 4 s (both switches are open). | Figure E 2.9-1 |
| --- | --- |

### Solution:

At  t = 4 s both switches are open, so i = 0 A.

| Exercise 2.9-2 What is the value of the voltage v in Figure E 2.9-2 at time t = 4 s? At t = 6 s? Answer: v = 6 volts at t = 4 s, and v = 0 volts at t = 6 s. | Figure E 2.9-2 |
| --- | --- |

### Solution:

At  t = 4 s the switch is in the up position, so .

At  t = 6 s the switch is in the down position, so v = 0 V.